| Model | AR61030 |  |  |
| --- | --- | --- | --- |
| Species |  |  | SCARA |
| Number of axes |  |  | 4 |
| Arm length |  |  | 600mm |
| Axis Specification | X Axis | Arm Length | 200mm |
|  |  | Rotation Range | ±145° |
|  | Y Axis | Arm Length | 400mm |
|  |  | Rotation Range | ±150° |
|  | Z Axis | Route | 300mm |
|  | R Axis | Rotation Range | ±360° |
| Highest Speed | X Axis |  | 300°/s |
|  | Y Axis |  | 375°/s |
|  | X,Y Axis Synthesis |  | 6.5m/s |
|  | Z Axis |  | 1.4m/s |
|  | R Axis |  | 1000°/s |
| Standard Cycle Time |  |  | 0.39s |
| Repeat Positioning Accuracy |  | X,Y Axis | ±0.003mm |
|  |  | Z Axis | ±0.02mm |
|  |  | R Axis | ±0.01° |
| Rated / Max Load |  |  | 10KG/20KG |
| R Axis Allow Load Moment of Inertia(Rated/Max) |  |  | 0.1 KG/㎡ |
|  |  |  | 0.5 KG/㎡ |
| User Signal Line |  |  | 0.2²×16Lines, 8 Output,6 Input |
| Users Pipe |  |  | φ6×2 Root |
| Limit Protection |  |  | 1.Software Limit 2.Mechanical Limit(X,Y,Z Axis) |
| Weight |  |  | 43.5KG |

AR61030机器人运动空间，外形及安装尺寸（单位：mm）：

图片模糊，请提供清晰的图片

特征与功能：

- 标准循环时间为0.39S

- 臂长600mm

- 额定/最大负载：10/20KG

- 适配控制系统QC400A/QC410，支持多种通讯方式：串口、网络、USB等

- 节省空间，性能优良，易于操作和编程，精度可达±0.02mm